Palak NFT Dataset Collection – Task & Implementation Plan

Step-by-step plan for sensor access, CP PLUS NVR camera access, ROI calibration, plant mapping and dataset collection

# 1. Objective

Build the raw-data collection pipeline for the Palak NFT experiment before training the Growth, Plant Health and Adaptive Nutrient Intelligence models. The system must reliably collect sensor readings and camera images, preserve original images, map every plant to a fixed NFT position, synchronize timestamps, and produce the four finalized operational data tables.

# 2. Current System

| Item | Current information | Task |
| --- | --- | --- |
| Crop | Palak | Confirm final planting/transplant date |
| NFT section | One dedicated section | Confirm exact number of channels and net pots |
| Camera | 6 MP, planned fixed top view | Confirm final mounting and actual stream quality |
| NVR | CP PLUS | Identify model and Palak camera channel |
| NVR IP | 192.168.100.35 | Verify reachability from collector/Raspberry Pi |
| Transport | TCP reported | Determine actual stream protocol, port and path |
| Sensors | Connected through Raspberry Pi | Identify current program/interface exposing readings |
| Network | College private LAN | Use an approved machine on the same reachable network |

# 3. Implementation Order

Stage 1 – Obtain Python access to live sensor data from the Raspberry Pi.

Stage 2 – Obtain Python access to the Palak camera stream through the CP PLUS NVR.

Stage 3 – Capture and validate one high-quality calibration image.

Stage 4 – Mark the NFT section, channels and individual net-pot/plant ROIs.

Stage 5 – Create the permanent plant registry.

Stage 6 – Automate full-section image capture and individual plant cropping.

Stage 7 – Continuously collect and timestamp sensor data.

Stage 8 – Synchronize image, plant, sensor and action records.

Stage 9 – Create the four operational data tables.

Stage 10 – Add growth/health annotations and later derive the three model datasets.

# 4. Stage 1 – Raspberry Pi Sensor Access

| Task | Success criterion |
| --- | --- |
| Identify Raspberry Pi IP/network | Collector can communicate with Pi |
| Identify current sensor program | Know whether data comes from Python, MQTT, database, file, serial, etc. |
| Identify available fields | pH, EC, water temperature, air temperature, humidity, water level, light if available |
| Read one live record in Python | Current sensor values print correctly |
| Timestamp readings | Every record has a reliable timestamp |
| Validate readings | Missing/invalid values are flagged |

Do not rewrite a working sensor-acquisition program unnecessarily. First create a Python interface around the existing system.

# 5. Stage 2 – CP PLUS NVR / Camera Access

| Task | What must be determined |
| --- | --- |
| Verify 192.168.100.35 is reachable | Network connection works |
| Identify NVR model | Exact CP PLUS model number |
| Identify Palak camera channel | NVR channel containing the Palak camera |
| Identify stream protocol | RTSP/HTTP/other supported local method |
| Identify port and path | Actual values; do not assume defaults |
| Identify main stream | Highest-quality stream for dataset capture |
| Check actual resolution | Saved frame resolution, not only 6 MP sensor rating |
| Check bitrate/FPS/encoding | Needed to diagnose unclear feed |
| Test Python frame capture | Python receives and saves a valid frame |

Because the 6 MP feed currently appears unclear, verify that the dataset collector is using the highest-quality/main stream rather than a low-resolution sub-stream.

# 6. Stage 3 – Camera Calibration Image

| Task | Output |
| --- | --- |
| Fix camera in final exact top-view position | Stable geometry |
| Record camera height/orientation | Calibration metadata |
| Capture complete NFT section | Original calibration image |
| Check all channels/net pots are visible | Complete coverage |
| Check individual plant detail | Crops are sufficiently clear |
| Preserve original image | Never overwrite source image |

# 7. Stage 4 – ROI Calibration

Because the camera is fixed, use one-time fixed Regions of Interest (ROIs). The calibration tool should allow marking the whole section, each channel and each individual net-pot/plant region.

| ROI level | Example ID | Purpose |
| --- | --- | --- |
| Whole section | S01 | Defines the experimental NFT section |
| Channel | C01, C02, ... | Defines each NFT channel |
| Net pot / plant position | C01-P01, C01-P02, ... | Defines each individual plant crop |

Save ROI coordinates to configuration. Do not manually redraw them for every future image.

# 8. Stage 5 – Plant Registry

| Field | Example | Purpose |
| --- | --- | --- |
| plant_id | PLANT_001 | Permanent biological plant identity |
| section_id | S01 | NFT section |
| channel_id | C01 | NFT channel |
| position_id | C01-P01 | Fixed physical net-pot position |
| crop | Palak | Crop identity |
| transplant_date | YYYY-MM-DD | Growth timeline |
| status | active | Active/replaced/harvested/dead |

If a plant is replaced, create a new biological plant_id while retaining the physical position_id.

# 9. Stage 6 – Automated Image Collection

| Task | Result |
| --- | --- |
| Capture scheduled camera frame | Original full-section image |
| Generate image_id | Unique image identifier |
| Record timestamp | Image can be synchronized |
| Save camera metadata | camera_id, section_id, resolution, quality |
| Apply saved ROIs | Plant crops created automatically |
| Attach plant IDs | Each crop mapped to its physical plant position |

# 10. Stage 7 – Continuous Sensor Collection

| Field | Example |
| --- | --- |
| timestamp | 2026-08-18 14:30:00 |
| section_id | S01 |
| reservoir_id | R01 |
| pH | 6.02 |
| EC | 1.12 mS/cm |
| water_temperature | 21.4 °C |
| air_temperature | 25.1 °C |
| humidity | 63 %RH |
| water_level | 82 % |
| light_intensity | 32000 lux |
| sensor_status | normal |

# 11. Stage 8 – Timestamp Synchronization

All streams must use a common clock. Preserve raw timestamps even when matching records using a time window.

| Record | Example |
| --- | --- |
| Sensor reading | 14:30:00 |
| Camera frame | 14:30:05 |
| Plant crop | Derived from 14:30:05 frame |
| Action | 14:15:00 |
| Response reading | 14:45:00 |

# 12. Stage 9 – Four Operational Tables

| Table | Purpose | Main content |
| --- | --- | --- |
| 1. Sensor / Environment | Raw time series | pH, EC, temperatures, humidity, water level, light |
| 2. Image / Camera | Original image metadata | image_id, timestamp, section, camera, path, resolution, quality |
| 3. Plant Observation | Plant-level history | plant_id, position, image, growth stage, health label, observations |
| 4. Actions / Response | Closed-loop intervention history | action, amount, timestamp, reason, before/after pH/EC, response |

# 13. Stage 10 – Human Annotation

| Annotation | Examples |
| --- | --- |
| Growth stage | Final categories to be defined consistently |
| Growth observation | Leaf count, canopy observation, visible growth change |
| Health label | Healthy or observed abnormality category |
| Notes | Human observation or experimental event |

# 14. Model Datasets – Created Later

| Model | Source tables | Candidate inputs/target |
| --- | --- | --- |
| Growth Intelligence | Image + Plant Observation | Plant image, time, growth observations → growth stage/trend |
| Plant Health Intelligence | Image + Plant Observation + Sensor | Plant image + environmental context → health label/risk |
| Adaptive Nutrient Intelligence | Sensor + Plant Observation + Actions/Response | Current/history + growth stage + previous actions → recommended action/response |

Future response variables such as next_EC/next_pH must not be used as inputs when predicting the preceding nutrient action.

# 15. Files to Create

| File | Purpose | When |
| --- | --- | --- |
| config/nft_layout.json | Section/channel/net-pot structure | Before ROI calibration |
| config/plant_registry.csv | Permanent plant/location mapping | Before collection |
| config/camera_config.json | NVR/camera settings | During camera setup |
| camera/camera_stream.py | Connect to CP PLUS stream | Stage 2 |
| camera/capture_images.py | Scheduled full-section capture | After stream test |
| roi/mark_rois.py | Interactive section/channel/net-pot marking | After camera test |
| roi/roi_manager.py | Load and manage saved ROIs | After calibration |
| plants/plant_cropper.py | Create individual plant crops | After ROI calibration |
| sensors/raspberry_pi_client.py | Read existing Pi sensor interface | Stage 1 |
| sensors/sensor_logger.py | Timestamp/store sensor readings | After sensor test |
| synchronization/synchronize_data.py | Join image/sensor/action observations | After raw collection works |
| annotation/growth_annotation.py | Growth labeling | During collection |
| annotation/health_annotation.py | Health labeling | During collection |
| dataset/build_growth_dataset.py | Create Growth dataset | Later |
| dataset/build_health_dataset.py | Create Health dataset | Later |
| dataset/build_nutrient_dataset.py | Create Nutrient dataset | Later |

# 16. Recommended Folder Structure

palak_dataset_collector/

config/ (nft_layout.json, plant_registry.csv, camera_config.json)

camera/ (camera_stream.py, capture_images.py)

roi/ (mark_rois.py, roi_manager.py)

plants/ (plant_cropper.py)

sensors/ (raspberry_pi_client.py, sensor_logger.py)

synchronization/ (synchronize_data.py)

annotation/ (growth_annotation.py, health_annotation.py)

dataset/ (build_growth_dataset.py, build_health_dataset.py, build_nutrient_dataset.py)

data/raw/sensors/

data/raw/images/

data/processed/full_section/

data/processed/plant_crops/

data/datasets/

logs/

# 17. Immediate Task Checklist

| Priority | Task | Completion condition |
| --- | --- | --- |
| 1 | Identify Raspberry Pi sensor program/interface | Python can access live sensor values |
| 2 | Verify Pi/collector can reach 192.168.100.35 | NVR reachable |
| 3 | Identify CP PLUS NVR model and Palak camera channel | Channel known |
| 4 | Identify actual stream protocol/port/path | Stream details known |
| 5 | Test Python sensor reader | Repeated valid readings |
| 6 | Test Python camera reader | Clear frame received/saved |
| 7 | Verify main-stream quality | Resolution/bitrate/FPS acceptable |
| 8 | Fix final camera top view | Whole NFT section visible |
| 9 | Run ROI marking | Section/channel/net-pot ROIs saved |
| 10 | Create plant registry | Every position has permanent IDs |
| 11 | Run image capture/cropping | Full images and plant crops created |
| 12 | Run continuous sensor logging | Sensor table grows correctly |
| 13 | Synchronize streams | Records can be joined by time/IDs |
| 14 | Begin growth/health annotation | Labels collected consistently |
| 15 | Build model datasets | Only after sufficient validated raw data |

# 18. Final Rule

The first goal is reliable raw-data collection, not model training. Preserve original images, raw sensor values, timestamps, fixed physical IDs, ROI definitions, actions and responses. The Growth, Health and Nutrient datasets should be derived from these validated records later.